Sink-Rate Measurement Checklist

Preparation Checks

- Weather

  - low inversion level (< 2000 ft)

  - low wind shear aloft

  - humidity aloft (test variable)

- Determine tow height needed

Cockpit Checks

- Battery charge

- Logger working

- Thermometer working

- Timer working

- Camcorder working and aimed

- Notepad and pens

  - speed-run sequence

  - temperatures

  - comments on data quality

- Same cockpit load/gross weight

Tow

- Retract gear at 1000 ft to speed high tows

- Tow upwind and away from commercial traffic lanes

Per Speed Run

- Record speed run number

- Record temperature

- Pitch up/down (notch the barograph) to identify start of run in IGC file

- Seek target speed

- Enunciate speed for camcorder

- Hold speed precisely for 45-60 seconds minimum quality time

- Record and enunciate quality issues

  - speed control issues

  - unsteady variometer

  - turbulence/shaking

- Cease taking data when air becomes rough